ETA9740 PCB LAYOUT GUIDE

Editor: ETA Shirley, Date: 2016-8-12

- Pin Configuration

- Pin6 BAT is not power pin ,while detection  single to battery.

- Pin 7 ,8,1 are power pins.

- LED1, LED2,LED3,ISET are common single.

- Schematic diagram for typical application

- PCB STACK

PCB typical thickness: 1.0mm

| Solder mask top | Cu | 0.5OZ/17.5um | sm001.pho |
| --- | --- | --- | --- |
| Top layer | Cu | 1.5OZ/52.5um | art001.pho |
| Dielectric CORE | FR4 | 860um |  |
| Solder mask bottom | Cu | 0.5OZ/17.5um | sm002.pho |
| Bottom layer | Cu | 1.5OZ/52.5um | art002.pho |

Note: 1OZ=35um=1.4mil

- PCB DESIGN RULE

|  | Trace | Via | Pad | Board |
| --- | --- | --- | --- | --- |
| Trace | 0.2mm/8mil | 0.2mm/8mil | 0.2mm/8mil | 0.25mm/10mil |
| Via |  | 0.2mm/8mil | 0.2mm/8mil | 0.25mm/10mil |
| Pad |  |  | 0.2mm/8mil | 0.25mm/10mil |
| Board |  |  |  | 0.25mm/10mil |

- VIA TYPE

| Via name |  |
| --- | --- |
| Single via | 0.4/0.6mm(16/24mil) or 0.3/0.5mm(12/20mil) |
| Power via (GND, VBUS, VOUT……) | 0.5/0.7mm(20/28mil) or 0.4/0.6mm(16/24mil) |

Note: Via to via can tangent but not overlap.

- PCB PLACEMENT

| LAYER | PLACEMENT | LAYOUT |
| --- | --- | --- |
| Top | Component | Power trace |
| Bottom | Cap , Res | Single trace& Power trace |

- OUT cap should be place close to IC pin 7&8, otherwise output need 2 caps with 0805 footprint and 1 cap with 0603 footprint.

- Inductance should place close to IC make sure that SW trace could trace as short as possible.

- Two caps are needed between inductance and battery welding point.

Bat trace must place from inductance to cap to welding point.

PS: Caps: 0805 cap &0603 cap

- Pin8 must connect to pin9

- Place a cap with 0805 footprint close to USBA

- ESOP pin9 needs some via dissipate heat

- Bat sense trace need far away from power current, it can put a via near the cap and go on the bottom side.

- Key need a 1206 Res series connection to output.

- PLACEMENT RULE DITAIL

- Power part:

- Component need to put compacted.

- Trace make sure that rout as short as possible.

- ETA9740 support 2.4A charge and discharge, so output trace rout wide>1.5mm

If the trace is larger than 25mm, please route wide>2.0mm.

- Single part:

- Bat sense need far away for power part.

- LED current limited res can place anywhere on the board.

- Layout EMI subjection

- Power current need more via on the board

- Power loop can be as small as possible.

- Can put some 0603 caps on the power trace for example: OUT……